An electronic structure descriptor for oxygen reactivity at metal and metal-oxide surfaces

Colin F. Dickens,1,2 Joseph H. Montoya,3 Ambarish R. Kulkarni,1,4 Michal Bajdich,2 and Jens K. Nørskov1,2,5*

1SUNCAT Center for Interface Science and Catalysis, Department of Chemical Engineering, Stanford University, Shriram Center, 443 Via Ortega, Stanford, California, 94305, USA.

2SUNCAT Center for Interface Science and Catalysis, SLAC National Accelerator Laboratory 2575 Sand Hill Road, Menlo Park, California, 94025, USA.

3Energy Technologies Area, Lawrence Berkeley National Laboratory, Berkeley, California, 94720, USA.

4Department of Chemical Engineering, University of California, Davis, California, 95616, USA.

5Department of Physics, Technical University of Denmark, 2800, Kongens Lyngby, Denmark.

*Correspondence to: jkno@dtu.dk

Supplementary Information

Computational Details

All DFT calculations in this work were performed using Quantum Espresso through the Atomic Simulation Environment interface.[1,2] Calculation parameters and optimization routines vary by dataset, so each will be explained separately in the following.

FCC metal calculations were performed using the BEEF-vdW exchange-correlation functional[3] with plane-wave/density-wave cutoffs of 600/6000 eV. Ultrasoft pseudopotentials from the GBRV library  were used.[4] Spin-polarization was neglected. The lattice constant for each metal was obtained by minimizing the energy of a bulk primitive cell with a 12 × 12 × 12 Monkhorst-Pack sampling of reciprocal space. Surface calculations were performed in a 3 × 3 × 3 cell of the (111) facet with the lower two layers fixed to their bulk positions and a 4 × 4 × 1 Monkhorst-Pack sampling of reciprocal space. A total vacuum region of 14 Å was added between periodic surfaces and a dipole correction was used. Local structural relaxations of surfaces were performed with a maximum force tolerance of 0.05 eV/Å. Additional structural constraints were applied to some of the FCC data points in Figure 1. For the cases of circles, diamonds, and triangles, the in-plane coordinates of oxygen in either adsorbed O or OH were fixed at an on-top, bridge, or FCC site, respectively. Star symbols indicate O and OH were relaxed at their most stable site without any constraints applied. For the structures in Figures 3, 5 and 6 the in-plane coordinates of oxygen and hydrogen were fixed at an on-top site and the perpendicular coordinate of oxygen was fixed at a particular value.

Rutile metal-oxide calculations were performed using the BEEF-vdW exchange-correlation functional with plane-wave/density-wave cutoffs of 600/6000 eV. Ultrasoft pseudopotentials from the GBRV library were used. Spin-polarization was neglected. The bulk lattice for each rutile metal-oxide from which the surface was cut was obtained by minimizing the energy of a bulk primitive cell (two formula units) with a 7 × 7 × 10 Monkhorst-Pack sampling of reciprocal space. Specifically, this minimization was performed with respect to the two lattice constants, a and c, and the reduced oxygen coordinate (often referred to as u). Surface calculations were performed in a 1 × 2 × 4 cell of the (110) facet with the lower two layers fixed to their bulk positions and a 4 × 4 × 1 Monkhorst-Pack sampling of reciprocal space. A total vacuum region of 12 Å was added between periodic surfaces and a dipole correction was used. Local structural relaxations of surfaces were performed with a maximum force tolerance of 0.05 eV/Å. For the rutile points in Figure 1, three oxygen sites were considered (as illustrated in the main text) and the stoichiometric surface (oxygen adsorbed at bridge and three-fold sites; nothing adsorbed at CUS) was always used as a starting point. Explicitly stated, the average O 2p-state energy of rutile circles is calculated from the DOS projected onto oxygen adsorbed at the CUS with nothing resting on the neighboring CUS and ΔEO–ΔEOH is calculated as the negative energy of adding a hydrogen atom to the same adsorbed oxygen. The average 2p-state energy of rutile diamonds and triangles is calculated from the DOS projected onto either the bridge or three-fold oxygen of a stoichiometric surface and ΔEO–ΔEOH is calculated as the negative energy of adding a hydrogen atom to the same either bridge or three-fold oxygen. The structure in Figure 7 is identical to the “CUS single-kink” from our previous work.[5]

The majority of perovskite data in Figure 1 directly utilize the calculations performed previously by Montoya et al. and the reader is directed to the original paper for details of those calculations.[6] Notable differences with the above data include the use of the RPBE exchange-correlation functional[7] and lower plane-wave/density-wave cutoffs of 400/4000 eV. We have reoptimized the structures of adsorbed OH and O for six surfaces with plane-wave/density-wave cutoffs of 600/6000 eV and saw negligible changes in the average O 2p-state energy and ΔEO–ΔEOH. In the perovskite calculations performed by Montoya et al., some distortions in the surface structure arose upon relaxation with adsorbates. We have not necessarily omitted cases where distortions arose, but in cases where there were significant relative distortions between adsorbed O and adsorbed OH, we re-relaxed the structures in an attempt to make the distortions more similar between states. Specifically, we removed hydrogen from the structure with adsorbed OH and performed a local relaxation using the same parameters as the original calculation. Ultimately, perovskite structures (both original calculations and those re-relaxed) were excluded from the dataset if the root-mean-square-displacement of all atoms between the structure with adsorbed O and the structure with adsorbed OH was greater than 0.15 Å. Perovskites in the final data-set that were re-relaxed by us as described above are indicated as such in Table S1.

Reference for O chemisorption

As mentioned in the main text, oxygen chemisorption energies (ΔEO) were calculated with respect to oxygen gas (in a triplet state). For the perovskite data set (which relies on RPBE), a standard +0.7 eV correction applied to the gas-phase reference (resulting in 0.35 eV stronger binding oxygen chemisorption energies) such that the energetics of water formation match experimental values. For the BEEF-vdW functional, it is somewhat ambiguous whether or not a correction should be applied because the functional was trained to experimental data using an uncorrected O2 reference. In this work we have chosen not to apply a correction to ΔEO for the datasets that rely on BEEF (rutile and FCC). This choice only affects the relative shift between perovskite data and rutile/FCC data, and correcting both RPBE and BEEF in the same manner results in a negligible change to the overall fit (no change in MAE to two decimal places).

Naturally, this choice of reference implies that OH chemisorption energies (ΔEOH) are with respect to oxygen gas and hydrogen gas.

Free energy correction for ΔEO–ΔEOH

The conventional descriptor for OER activity is ΔGO–ΔGOH at standard conditions (300 K, 1 bar of hydrogen gas), which differs from ΔEO–ΔEOH by vibrational energy and entropy of the surface and translational, rotational, and vibrational energy and entropy of hydrogen gas:

Using the IdealGasThermo class in the thermochemistry module of the Atomic Simulation Environment (ASE), we calculate the free energy correction for ½H2 gas to be -0.02 eV, and using the HarmonicThermo class in (ASE), we have calculated the free energy corrections for either adsorbed O or OH and the difference between them for 178 of the data points in Figure 1, as shown in the histogram in Figure S8. The mean +/- one standard deviation for the free energy corrections of O, OH, and O-OH are 0.05 +/- 0.02 eV, 0.32 +/- 0.04 eV, -0.27 +/- 0.03 eV, respectively. Therefore, the average total free energy correction to be applied to ΔEO–ΔEOH is -0.27 – 0.02 = -0.29 eV.

Iterative scheme to determine most stable surface coverage

It is often desirable to calculate catalytic properties with the appropriate coverage under reaction conditions. In principle, the most energetically stable coverage (considering either adsorbed OH or O) could be predicted based on the ε2p of each oxygen (which yields an estimate of ΔEO–ΔEOH) given the chemical potential of hydrogen. However, if adsorbate-adsorbate interactions are significant, then the reactivity of each oxygen may change upon addition of hydrogen to the surface. The change in reactivity could of course be estimated by performing an additional self-consistent DFT calculation of the new surface coverage and extracting ε2p of each oxygen site, and the process could be iterated on until the convergence is achieved. We show preliminary evidence in Figure S7 that suggests ε2p of oxygen in adsorbed OH could be used to predict the energetics of its deprotonation, which would be necessary in such a scheme.

Numerical Methods

Integration of the DOS spectra was performed via the trapezoidal rule as implemented in the python package NumPy.

Raw Data

All structures and DOS considered in this study are available in a zip file. Structures are provided either in CUBE or ASE-Trajectory formats (the latter includes some calculation information including structural constraints), and DOS is provided in a csv with up to three columns: energy relative to the Fermi level, DOS projected onto 2p-states of the oxygen atom of interest, total DOS. Each data point in this study is represented by 2-3 structures (one for O adsorbed, one for OH adsorbed and in some cases, one for an empty site) and a DOS spectrum projected onto O. The naming scheme for the structures used in Figures 1, S2, and S3 is chemical_formula-coordination-adsorbate. For example, LaOsO3-2c-OH.traj represents OH adsorbed with two-fold coordination to Os (i.e. in the OsO2 plane) on LaOsO3, LaOsO3-2c-s.traj has the same structure with the OH removed (leaving an oxygen vacancy), and LaOsO3-2c-O.traj replenishes the vacancy with an oxygen atom. Analogously, the DOS projected onto the adsorbed oxygen for the latter structure is contained in LaOsO3-2c-O.csv. The naming scheme for data in Figure 3 is metal-bond_length-species (e.g. Ag-2.60-O.traj) where the M-O bond length is in units of Å. The naming scheme for data in Figure 7 is RuO2_kink-atom_index-adsorbate (e.g. RuO2_kink-6-OH.traj) where atom_index is the index of the oxygen site of interest (zero-based indexing).

Structures and energies are also available for viewing at CatalysisHub.org.[8]

Figure S1. Sensitivity of Figure 1 correlation with respect to the integration upper bound. MAE and RMSE are the mean absolute error and root mean absolute error, respectively, of the linear model.

Figure S2. Analogous to Figure 1 of the main text but including the oxygen chemisorption energy relative to O2(g). Note that the perovskite structures in this study are taken from ref. [6], where calculations of bridge-site oxygen vacancies were not performed (necessary for calculating oxygen chemisorption at bridge sites). This accounts for the smaller data set analyzed in this figure compared to Figure 1.

Figure S3. Analogous to Figure 1 of the main text but including the OH chemisorption energy relative to O2(g) and H2(g).

Figure S4. O-H and M-O bond lengths for the data in Figure 1 of the main text. M-O bond lengths correspond to the structure after deprotonation (O adsorbed).

Figure S5. Correlation between the d-band center and either the oxygen chemisorption energy (unfilled) or the energy required to deprotonate surface OH (filled) for surfaces in Figure 3.

Figure S6. Correlation between either ΔEO–ΔEOH (filled) or ΔEOH (unfilled) and ε2p for all grid elements in Figure 3 of the main text.

Figure S7. Correlation between the energy required to deprotonate adsorbed OH and the average 2p-state energy of oxygen in adsorbed OH for each site considered in Figure 7.

Figure S8. Histogram of free energy corrections for 178 of the data points in Figure 1 of the main text.

| Table S1. Raw data from Figures 1, S2, and S3. Asterisks (*) next to perovskite names indicate they were re-relaxed as explained above. FCC entries without coordination numbers indicate that O and OH were relaxed without constraints on their most stable respective sites. All energies are in units of eV. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Formula | Coordination |  | ΔEO–ΔEOH | ΔEO | ΔEOH |
| Ag | 1 | -1.00 | 2.31 | 1.03 | -1.29 |
| Ag | 2 | -1.89 | 1.56 | -0.13 | -1.69 |
| Ag | 3 | -2.02 | 1.44 | -0.41 | -1.85 |
| Au | 1 | -1.25 | 2.28 | 1.28 | -1 |
| Au | 3 | -2.62 | 1.17 | -0.02 | -1.19 |
| Cd | 1 | -1.13 | 2.05 | 0.35 | -1.71 |
| Cd | 2 | -2.91 | 0.62 | -1.58 | -2.2 |
| Cd | 3 | -2.81 | 0.73 | -1.57 | -2.29 |
| Co | 1 | -2.49 | 1.11 | -1.02 | -2.13 |
| Co | 2 | -3.71 | 0.14 | -2.29 | -2.43 |
| Co | 3 | -4.08 | -0.09 | -2.83 | -2.75 |
| Cr | 3 | -4.67 | -0.37 | -3.31 | -2.94 |
| Cu | 1 | -1.57 | 1.81 | 0.10 | -1.71 |
| Cu | 2 | -2.91 | 0.84 | -1.27 | -2.11 |
| Cu | 3 | -3.10 | 0.67 | -1.62 | -2.29 |
| Fe | 1 | -3.02 | 0.65 | -1.53 | -2.18 |
| Fe | 2 | -4.00 | -0.07 | -2.65 | -2.58 |
| Fe | 3 | -4.14 | -0.15 | -3.03 | -2.88 |
| Ir | 1 | -2.60 | 1.15 | -0.84 | -1.98 |
| Ir | 2 | -3.28 | 0.71 | -1.27 | -1.97 |
| Ir | 3 | -3.77 | 0.13 | -1.73 | -1.86 |
| Mn | 1 | -3.04 | 0.09 | -2.53 | -2.62 |
| Mn | 2 | -4.20 | -0.41 | -3.48 | -3.08 |
| Mn | 3 | -4.26 | -0.35 | -3.70 | -3.35 |
| Mo | 1 | -3.48 | -0.10 | -3.07 | -2.98 |
| Nb | 1 | -2.98 | 0.36 | -4.68 | -5.04 |
| Nb | 3 | -4.38 | -0.48 | -6.57 | -6.09 |
| Ni | 1 | -2.14 | 1.43 | -0.57 | -2 |
| Ni | 2 | -3.53 | 0.32 | -1.93 | -2.25 |
| Ni | 3 | -3.96 | 0.05 | -2.47 | -2.52 |
| Os | 1 | -3.48 | 0.72 | -1.35 | -2.06 |
| Os | 2 | -3.86 | 0.48 | -1.69 | -2.17 |
| Os | 3 | -4.23 | 0.03 | -2.02 | -2.06 |
| Pb | 1 | -1.29 | 1.75 | -0.28 | -2.03 |
| Pb | 2 | -2.46 | 0.99 | -1.20 | -2.19 |
| Pd | 1 | -1.73 | 1.85 | 0.31 | -1.54 |
| Pd | 2 | -2.81 | 1.03 | -0.70 | -1.73 |
| Pd | 3 | -3.35 | 0.57 | -1.16 | -1.73 |
| Pt | 1 | -2.14 | 1.67 | 0.03 | -1.64 |
| Pt | 2 | -3.14 | 0.88 | -0.71 | -1.59 |
| Pt | 3 | -3.85 | 0.22 | -1.23 | -1.44 |
| Re | 1 | -3.75 | -0.15 | -2.65 | -2.49 |
| Re | 2 | -4.40 | -0.33 | -2.76 | -2.43 |
| Re | 3 | -4.57 | -0.40 | -3.06 | -2.66 |
| Rh | 1 | -2.29 | 1.34 | -0.58 | -1.91 |
| Rh | 2 | -3.17 | 0.75 | -1.38 | -2.14 |
| Rh | 3 | -3.56 | 0.36 | -1.81 | -2.17 |
| Ru | 1 | -3.15 | 0.80 | -1.24 | -2.04 |
| Ru | 2 | -3.68 | 0.39 | -2.00 | -2.39 |
| Ru | 3 | -3.84 | 0.11 | -2.30 | -2.41 |
| Sc | 1 | -2.18 | 0.60 | -3.47 | -4.07 |
| Sc | 2 | -4.00 | -0.65 | -5.49 | -4.84 |
| Sc | 3 | -4.34 | -1.00 | -6.13 | -5.13 |
| Sn | 1 | -1.52 | 1.52 | -0.78 | -2.3 |
| Sn | 2 | -3.17 | 0.50 | -1.61 | -2.12 |
| Sn | 3 | -4.05 | 0.12 | -1.92 | -2.05 |
| Ti | 1 | -2.98 | 0.31 | -3.42 | -3.73 |
| Ti | 2 | -4.33 | -0.62 | -5.23 | -4.61 |
| Ti | 3 | -4.76 | -0.97 | -5.92 | -4.95 |
| V | 3 | -4.79 | -0.56 | -3.35 | -2.79 |
| W | 1 | -3.50 | -0.15 | -5.06 | -4.91 |
| W | 3 | -5.13 | -0.37 | -4.37 | -4.01 |
| Y | 1 | -1.88 | 0.85 | -3.15 | -4 |
| Y | 2 | -3.75 | -0.56 | -5.29 | -4.73 |
| Y | 3 | -4.16 | -0.97 | -6.01 | -5.04 |
| Zn | 1 | -1.53 | 1.97 | 0.04 | -1.93 |
| Zn | 3 | -3.58 | 0.29 | -2.04 | -2.32 |
| Zr | 1 | -2.62 | 0.54 | -3.16 | -3.69 |
| Zr | 2 | -4.20 | -0.62 | -5.08 | -4.47 |
| Zr | 3 | -4.71 | -1.03 | -5.87 | -4.84 |
| Ag | N/A | -2.01 | 1.44 | -0.41 | -1.85 |
| Au | N/A | -2.62 | 1.17 | -0.02 | -1.19 |
| Cd | N/A | -2.91 | 0.70 | -1.60 | -2.3 |
| Co | N/A | -4.08 | -0.08 | -2.83 | -2.75 |
| Cr | N/A | -4.61 | -1.27 | -4.40 | -3.13 |
| Cu | N/A | -3.10 | 0.67 | -1.62 | -2.29 |
| Fe | N/A | -4.14 | -0.15 | -3.03 | -2.88 |
| Ir | N/A | -3.76 | 0.26 | -1.73 | -1.99 |
| Mn | N/A | -4.25 | -0.35 | -3.70 | -3.35 |
| Mo | N/A | -4.72 | -0.60 | -4.20 | -3.6 |
| Nb | N/A | -4.62 | -0.68 | -7.44 | -6.76 |
| Ni | N/A | -3.94 | 0.05 | -2.47 | -2.52 |
| Os | N/A | -4.23 | 0.14 | -2.02 | -2.17 |
| Pb | N/A | -3.07 | 0.71 | -1.50 | -2.21 |
| Pd | N/A | -3.35 | 0.57 | -1.16 | -1.73 |
| Pt | N/A | -3.85 | 0.42 | -1.23 | -1.65 |
| Re | N/A | -4.57 | -0.40 | -3.06 | -2.66 |
| Rh | N/A | -3.57 | 0.36 | -1.81 | -2.17 |
| Ru | N/A | -3.83 | 0.11 | -2.30 | -2.41 |
| Sc | N/A | -4.35 | -1.00 | -6.13 | -5.13 |
| Sn | N/A | -4.05 | 0.26 | -1.93 | -2.18 |
| Ti | N/A | -4.76 | -0.97 | -5.92 | -4.95 |
| V | N/A | -4.63 | -0.65 | -5.07 | -4.43 |
| W | N/A | -5.04 | -1.08 | -6.22 | -5.14 |
| Y | N/A | -4.17 | -0.97 | -6.01 | -5.04 |
| Zn | N/A | -3.58 | 0.29 | -2.04 | -2.32 |
| Zr | N/A | -4.70 | -1.03 | -5.87 | -4.84 |
| GeO2 | 1 | -0.85 | 3.35 | 2.51 | -0.84 |
| GeO2 | 2 | -2.53 | 0.43 | -2.81 | -3.24 |
| GeO2 | 3 | -4.53 | -0.99 | -3.13 | -2.15 |
| IrO2 | 1 | -2.44 | 1.66 | -1.00 | -2.66 |
| IrO2 | 2 | -2.91 | 1.21 | -2.56 | -3.77 |
| IrO2 | 3 | -4.63 | -0.09 | -3.05 | -2.96 |
| MoO2 | 1 | -3.58 | 0.24 | -2.84 | -3.08 |
| NbO2 | 1 | -3.55 | 0.04 | -3.53 | -3.57 |
| NbO2 | 3 | -4.75 | -1.05 | -5.11 | -4.06 |
| PbO2 | 1 | -0.58 | 3.29 | 2.92 | -0.37 |
| PbO2 | 2 | -1.27 | 2.43 | 0.53 | -1.89 |
| PbO2 | 3 | -2.32 | 1.40 | -0.21 | -1.62 |
| PtO2 | 1 | -1.83 | 2.39 | 0.47 | -1.92 |
| PtO2 | 2 | -2.29 | 1.61 | -1.24 | -2.84 |
| PtO2 | 3 | -3.94 | 0.42 | -2.00 | -2.42 |
| RhO2 | 1 | -2.02 | 2.09 | 0.31 | -1.78 |
| RhO2 | 2 | -2.35 | 1.79 | -1.30 | -3.09 |
| RhO2 | 3 | -3.67 | 0.68 | -2.36 | -3.04 |
| RuO2 | 1 | -2.84 | 1.27 | -0.98 | -2.25 |
| RuO2 | 2 | -3.38 | 1.04 | -2.48 | -3.52 |
| RuO2 | 3 | -4.19 | -0.13 | -3.32 | -3.19 |
| SnO2 | 1 | -0.62 | 3.36 | 2.43 | -0.92 |
| SnO2 | 2 | -1.67 | 1.28 | -2.36 | -3.63 |
| SnO2 | 3 | -3.72 | -0.20 | -2.94 | -2.74 |
| TaO2 | 1 | -3.37 | 0.02 | -4.29 | -4.32 |
| TaO2 | 2 | -5.14 | -0.75 | -5.34 | -4.59 |
| TaO2 | 3 | -5.94 | -1.71 | -4.79 | -3.08 |
| TiO2 | 1 | -0.74 | 2.85 | 2.08 | -0.76 |
| TiO2 | 2 | -3.09 | 0.09 | -4.11 | -4.2 |
| TiO2 | 3 | -3.62 | -0.55 | -5.25 | -4.7 |
| WO2 | 1 | -3.80 | 0.18 | -3.73 | -3.91 |
| WO2 | 3 | -5.23 | -1.38 | -3.61 | -2.23 |
| BaAgO3 | 2 | -1.37 | 2.63 | N/A | N/A |
| BaAlO3 | 2 | -1.58 | 2.43 | N/A | N/A |
| BaAsO3 | 2 | -2.13 | 2.27 | N/A | N/A |
| BaBO3 | 2 | -5.21 | -0.53 | N/A | N/A |
| BaBaO3 | 2 | -0.89 | 1.65 | N/A | N/A |
| BaCuO3 | 2 | -1.41 | 2.60 | N/A | N/A |
| BaGaO3 | 2 | -1.44 | 2.56 | N/A | N/A |
| BaGeO3 | 2 | -2.25 | 1.87 | N/A | N/A |
| BaHfO3 | 2 | -2.55 | -1.27 | N/A | N/A |
| BaHgO3 | 2 | -1.52 | 2.37 | N/A | N/A |
| BaInO3 | 2 | -1.30 | 2.69 | N/A | N/A |
| BaIrO3 | 2 | -3.29 | 0.96 | N/A | N/A |
| BaPtO3 | 2 | -2.48 | 1.85 | N/A | N/A |
| BaReO3 | 2 | -4.57 | -0.46 | N/A | N/A |
| BaScO3 | 2 | -1.06 | 2.57 | N/A | N/A |
| BaSnO3 | 2 | -2.09 | 1.49 | N/A | N/A |
| BaTaO3 | 2 | -5.04 | -1.92 | N/A | N/A |
| BaTiO3 | 2 | -2.37 | 0.87 | N/A | N/A |
| BaZrO3 | 2 | -2.46 | -1.04 | N/A | N/A |
| CaAlO3 | 1 | -0.75 | 3.04 | 2.35 | -0.7 |
| CaBiO3 | 1 | -0.80 | 2.30 | 0.10 | -2.2 |
| CaCuO3 | 1 | -1.01 | 2.76 | 1.86 | -0.91 |
| CaGaO3 | 1 | -0.66 | 3.14 | 2.40 | -0.74 |
| CaGeO3 | 1 | -0.96 | 3.06 | 1.21 | -1.84 |
| CaNbO3 | 1 | -3.98 | -0.56 | -5.04 | -4.48 |
| CaNiO3 | 1 | -1.58 | 2.36 | -0.25 | -2.61 |
| CaScO3 | 1 | -0.33 | 3.25 | 2.31 | -0.94 |
| CaSiO3 | 1 | -1.08 | 3.11 | 0.78 | -2.33 |
| CaSiO3 | 2 | -3.68 | -0.56 | N/A | N/A |
| CaTaO3 | 1 | -4.06 | -0.71 | -5.80 | -5.09 |
| CaTiO3 | 1 | -0.66 | 2.68 | 1.18 | -1.49 |
| CaYO3 | 1 | -0.27 | 3.25 | 1.81 | -1.44 |
| KBiO3 | 2 | -1.58 | 2.36 | N/A | N/A |
| KCuO3 | 2 | -1.21 | 2.72 | N/A | N/A |
| KHfO3 | 2 | -1.33 | 2.38 | N/A | N/A |
| KOsO3 | 2 | -3.90 | 0.27 | N/A | N/A |
| KRuO3 | 2 | -2.55 | 1.54 | N/A | N/A |
| KSbO3 | 2 | -2.65 | 1.24 | N/A | N/A |
| KSnO3 | 2 | -1.57 | 2.48 | N/A | N/A |
| KTaO3 | 2 | -4.55 | -1.24 | N/A | N/A |
| KTiO3 | 2 | -1.43 | 2.67 | N/A | N/A |
| KZrO3 | 2 | -1.23 | 2.45 | N/A | N/A |
| LaAlO3 | 1 | -0.65 | 3.21 | 2.27 | -0.94 |
| LaCuO3 | 1 | -0.95 | 2.82 | 1.93 | -0.88 |
| LaGaO3 | 2 | -1.89 | 2.28 | N/A | N/A |
| LaIrO3 | 1 | -2.63 | 1.41 | -2.41 | -3.82 |
| LaMgO3 | 1 | -0.28 | 3.34 | 2.60 | -0.74 |
| LaNiO3 | 1 | -1.50 | 2.39 | 0.85 | -1.55 |
| LaReO3 | 1 | -4.32 | -0.25 | -4.23 | -3.98 |
| LaTiO3 | 2 | -4.97 | -1.56 | N/A | N/A |
| LaZnO3 | 1 | -0.71 | 3.06 | 2.50 | -0.56 |
| NaHfO3 | 2 | -1.34 | 2.44 | N/A | N/A |
| NaOsO3 | 2 | -4.20 | 0.24 | N/A | N/A |
| NaSiO3 | 2 | -2.62 | 1.94 | N/A | N/A |
| NaSnO3 | 2 | -1.57 | 2.53 | N/A | N/A |
| NaTaO3 | 2 | -4.75 | -1.29 | N/A | N/A |
| NaTiO3 | 2 | -1.57 | 2.43 | N/A | N/A |
| NaZrO3 | 2 | -1.24 | 2.51 | N/A | N/A |
| SrAgO3 | 1 | -0.90 | 2.91 | 2.12 | -0.79 |
| SrAlO3 | 1 | -0.80 | 3.04 | 2.25 | -0.79 |
| SrAlO3 | 2 | -1.64 | 2.33 | N/A | N/A |
| SrAuO3 | 1 | -1.27 | 2.78 | 1.71 | -1.06 |
| SrGaO3 | 1 | -0.73 | 3.11 | 2.38 | -0.73 |
| SrGeO3 | 1 | -1.04 | 3.04 | 1.22 | -1.83 |
| SrGeO3 | 2 | -2.46 | 1.59 | N/A | N/A |
| SrHfO3 | 1 | -0.44 | 3.22 | 1.53 | -1.69 |
| SrHfO3 | 2 | -2.54 | -1.04 | N/A | N/A |
| SrIrO3 | 1 | -2.86 | 1.52 | -0.75 | -2.27 |
| SrMoO3 | 1 | -3.37 | 0.06 | -3.54 | -3.59 |
| SrNbO3 | 1 | -3.89 | -0.42 | -4.79 | -4.37 |
| SrPbO3 | 1 | -0.70 | 2.95 | 1.56 | -1.4 |
| SrPdO3 | 1 | -1.45 | 2.38 | 0.74 | -1.64 |
| SrReO3 | 1 | -3.89 | -0.09 | -4.24 | -4.16 |
| SrScO3 | 1 | -0.34 | 3.24 | 2.48 | -0.75 |
| SrSnO3 | 1 | -0.70 | 3.13 | 1.41 | -1.72 |
| SrSnO3 | 2 | -2.29 | 1.44 | N/A | N/A |
| SrTaO3 | 1 | -3.95 | -0.60 | -5.68 | -5.08 |
| SrTiO3 | 1 | -0.72 | 2.69 | 1.03 | -1.66 |
| SrTlO3 | 1 | -0.76 | 3.00 | 2.10 | -0.91 |
| SrWO3 | 1 | -3.78 | -0.39 | -4.89 | -4.5 |
| SrYO3 | 1 | -0.28 | 3.25 | 1.90 | -1.35 |
| SrZnO3 | 1 | -0.79 | 3.00 | 2.48 | -0.52 |
| SrZrO3 | 1 | -0.46 | 3.14 | 1.50 | -1.64 |
| YAlO3 | 2 | -2.35 | 1.93 | N/A | N/A |
| YRhO3 | 2 | -3.02 | 0.92 | N/A | N/A |
| CaAgO3* | 1 | -0.84 | 2.83 | 1.93 | -0.9 |
| CaAgO3* | 2 | -1.29 | 2.54 | N/A | N/A |
| CaAlO3* | 2 | -1.76 | 2.25 | N/A | N/A |
| CaAuO3* | 1 | -1.19 | 2.67 | 1.41 | -1.26 |
| CaBO3* | 1 | -1.94 | 1.91 | N/A | N/A |
| CaBaO3* | 2 | -1.13 | 1.20 | N/A | N/A |
| CaCuO3* | 2 | -1.46 | 2.48 | N/A | N/A |
| CaGaO3* | 2 | -1.60 | 2.56 | N/A | N/A |
| CaGeO3* | 2 | -2.64 | 1.13 | N/A | N/A |
| CaInO3* | 2 | -1.06 | 2.88 | N/A | N/A |
| CaIrO3* | 1 | -2.79 | 1.40 | N/A | N/A |
| CaLaO3* | 2 | -0.98 | 2.49 | N/A | N/A |
| CaPbO3* | 1 | -0.69 | 3.05 | 1.76 | -1.3 |
| CaPbO3* | 2 | -1.36 | 2.13 | N/A | N/A |
| CaReO3* | 1 | -3.92 | -0.06 | N/A | N/A |
| CaRhO3* | 1 | -2.42 | 1.66 | N/A | N/A |
| CaRhO3* | 2 | -2.51 | 1.65 | N/A | N/A |
| CaScO3* | 2 | -0.92 | 2.73 | N/A | N/A |
| CaSnO3* | 1 | -0.69 | 3.16 | 1.51 | -1.65 |
| CaSnO3* | 2 | -2.04 | 1.33 | N/A | N/A |
| CaTaO3* | 2 | -5.13 | -1.55 | N/A | N/A |
| CaTeO3* | 1 | -1.43 | 2.04 | N/A | N/A |
| CaTiO3* | 2 | -2.74 | 0.11 | N/A | N/A |
| CaTlO3* | 1 | -0.74 | 3.04 | 2.00 | -1.04 |
| CaWO3* | 1 | -4.00 | -0.49 | N/A | N/A |
| CaYO3* | 2 | -0.64 | 2.79 | N/A | N/A |
| CaZrO3* | 1 | -0.45 | 3.15 | 1.89 | -1.27 |
| CaZrO3* | 2 | -2.18 | -0.88 | N/A | N/A |
| LaAgO3* | 1 | -0.83 | 2.84 | 2.01 | -0.83 |
| LaAlO3* | 2 | -1.98 | 1.82 | N/A | N/A |
| LaAsO3* | 1 | -1.73 | 2.16 | N/A | N/A |
| LaAsO3* | 2 | -2.15 | 1.38 | N/A | N/A |
| LaAuO3* | 1 | -1.18 | 2.57 | 1.50 | -1.07 |
| LaBO3* | 2 | -1.85 | 2.02 | N/A | N/A |
| LaCaO3* | 1 | -0.43 | 3.00 | 0.39 | -2.61 |
| LaCdO3* | 1 | -0.52 | 3.19 | 2.26 | -0.93 |
| LaCoO3* | 1 | -1.85 | 1.88 | 0.09 | -1.79 |
| LaCoO3* | 2 | -2.47 | 1.44 | N/A | N/A |
| LaCuO3* | 2 | -1.87 | 1.88 | N/A | N/A |
| LaHfO3* | 1 | -2.89 | 0.01 | N/A | N/A |
| LaHgO3* | 1 | -1.02 | 2.88 | 1.43 | -1.45 |
| LaIrO3* | 2 | -3.73 | 0.81 | N/A | N/A |
| LaMgO3* | 2 | -0.98 | 2.82 | N/A | N/A |
| LaNbO3* | 1 | -4.12 | -0.94 | N/A | N/A |
| LaNiO3* | 2 | -2.26 | 1.75 | N/A | N/A |
| LaOsO3* | 1 | -3.46 | 0.41 | -3.09 | -3.5 |
| LaOsO3* | 2 | -5.14 | -0.22 | N/A | N/A |
| LaPbO3* | 1 | -0.64 | 2.65 | N/A | N/A |
| LaPdO3* | 1 | -1.36 | 2.42 | 0.67 | -1.75 |
| LaPdO3* | 2 | -2.06 | 1.60 | N/A | N/A |
| LaPtO3* | 1 | -1.81 | 2.21 | 0.05 | -2.16 |
| LaPtO3* | 2 | -3.16 | 1.07 | N/A | N/A |
| LaRhO3* | 1 | -2.23 | 1.73 | -0.68 | -2.41 |
| LaRhO3* | 2 | -3.07 | 0.92 | N/A | N/A |
| LaRuO3* | 1 | -2.47 | 1.31 | -0.87 | -2.17 |
| LaRuO3* | 2 | -3.54 | 0.52 | N/A | N/A |
| LaSiO3* | 2 | -4.26 | -0.87 | N/A | N/A |
| LaTaO3* | 1 | -4.26 | -0.94 | N/A | N/A |
| LaTaO3* | 2 | -4.80 | -1.58 | N/A | N/A |
| LaTlO3* | 1 | -0.68 | 3.07 | 1.83 | -1.24 |
| LaZnO3* | 2 | -1.78 | 2.17 | N/A | N/A |
| LaZrO3* | 1 | -3.26 | -0.30 | N/A | N/A |
| SrAgO3* | 2 | -1.23 | 2.62 | N/A | N/A |
| SrBO3* | 2 | -5.27 | -0.60 | N/A | N/A |
| SrGaO3* | 2 | -1.46 | 2.48 | N/A | N/A |
| SrIrO3* | 2 | -3.62 | 1.41 | N/A | N/A |
| SrNiO3* | 2 | -2.70 | 2.06 | N/A | N/A |
| SrPbO3* | 2 | -1.37 | 2.24 | N/A | N/A |
| SrReO3* | 2 | -4.52 | -0.45 | N/A | N/A |
| SrRhO3* | 1 | -2.27 | 1.86 | N/A | N/A |
| SrRhO3* | 2 | -2.57 | 1.46 | N/A | N/A |
| SrRuO3* | 1 | -2.31 | 1.38 | -0.70 | -2.08 |
| SrSbO3* | 1 | -1.14 | 2.09 | -0.79 | -2.87 |
| SrScO3* | 2 | -1.10 | 2.52 | N/A | N/A |
| SrSiO3* | 2 | -3.32 | 0.17 | N/A | N/A |
| SrTaO3* | 2 | -5.28 | -2.06 | N/A | N/A |
| SrTiO3* | 2 | -2.51 | 0.18 | N/A | N/A |
| SrTlO3* | 2 | -1.07 | 2.62 | N/A | N/A |
| SrYO3* | 2 | -0.70 | 2.81 | N/A | N/A |
| SrZnO3* | 2 | -1.43 | 2.50 | N/A | N/A |
| SrZrO3* | 2 | -2.46 | -0.89 | N/A | N/A |

| Table S2. Raw data from Figures 5 and S6. Bond lengths are in units of Å and all energies are in units of eV. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Formula | M-O Length |  | ΔEO–ΔEOH | ΔEO | ΔEOH |
| Au | 2.08 | -0.92 | 1.76 | 1.43 | -0.33 |
| Au | 2.25 | -0.64 | 2.06 | 1.78 | -0.28 |
| Au | 2.36 | -0.51 | 2.23 | 2.04 | -0.19 |
| Au | 2.38 | -0.48 | 2.26 | 2.08 | -0.18 |
| Mo | 2.08 | -1.64 | 1.04 | -1.39 | -2.43 |
| Mo | 2.25 | -1.16 | 1.5 | -0.6 | -2.1 |
| Mo | 2.36 | -0.93 | 1.75 | -0.09 | -1.84 |
| Mo | 2.38 | -0.89 | 1.79 | -0.01 | -1.8 |
| Ru | 2.08 | -1.37 | 1.31 | -0.25 | -1.56 |
| Ru | 2.25 | -0.92 | 1.74 | 0.43 | -1.31 |
| Ru | 2.36 | -0.72 | 1.96 | 0.86 | -1.1 |
| Ru | 2.38 | -0.69 | 1.99 | 0.94 | -1.05 |
| Ti | 2.08 | -1.52 | 1.07 | -2.21 | -3.28 |
| Ti | 2.25 | -1.14 | 1.44 | -1.37 | -2.81 |
| Ti | 2.36 | -0.95 | 1.65 | -0.85 | -2.5 |
| Ti | 2.38 | -0.92 | 1.69 | -0.75 | -2.44 |

| Table S3. Raw data from Figure 6. Bond lengths are in units of Å and all energies are in units of eV. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Formula | M-O Length |  | ΔEO–ΔEOH | ΔEO | ΔEOH |
| Ti | 1.22 | -7.07 | -3.14 | 5.25 | 8.39 |
| Ti | 1.24 | -6.84 | -2.98 | 4.24 | 7.22 |
| Ti | 1.26 | -6.61 | -2.8 | 3.32 | 6.12 |
| Ti | 1.28 | -6.39 | -2.63 | 2.49 | 5.12 |
| Ti | 1.3 | -6.18 | -2.46 | 1.75 | 4.21 |
| Ti | 1.32 | -5.97 | -2.3 | 1.07 | 3.37 |
| Ti | 1.34 | -5.77 | -2.14 | 0.46 | 2.6 |
| Ti | 1.36 | -5.57 | -1.99 | -0.08 | 1.91 |
| Ti | 1.38 | -5.38 | -1.84 | -0.57 | 1.27 |
| Ti | 1.4 | -5.19 | -1.7 | -1.01 | 0.69 |
| Ti | 1.42 | -5.01 | -1.56 | -1.4 | 0.16 |
| Ti | 1.44 | -4.84 | -1.43 | -1.74 | -0.31 |
| Ti | 1.46 | -4.67 | -1.3 | -2.04 | -0.74 |
| Ti | 1.48 | -4.51 | -1.18 | -2.31 | -1.13 |
| Ti | 1.5 | -4.35 | -1.07 | -2.54 | -1.47 |
| Ti | 1.52 | -4.2 | -0.95 | -2.74 | -1.79 |
| Ti | 1.54 | -4.05 | -0.85 | -2.91 | -2.06 |
| Ti | 1.56 | -3.91 | -0.74 | -3.05 | -2.31 |
| Ti | 1.58 | -3.77 | -0.64 | -3.17 | -2.53 |
| Ti | 1.6 | -3.64 | -0.55 | -3.26 | -2.71 |
| Ti | 1.62 | -3.51 | -0.45 | -3.34 | -2.89 |
| Ti | 1.64 | -3.39 | -0.37 | -3.39 | -3.02 |
| Ti | 1.66 | -3.27 | -0.28 | -3.43 | -3.15 |
| Ti | 1.68 | -3.15 | -0.19 | -3.46 | -3.27 |
| Ti | 1.7 | -3.04 | -0.11 | -3.46 | -3.35 |
| Ti | 1.72 | -2.93 | -0.03 | -3.46 | -3.43 |
| Ti | 1.74 | -2.82 | 0.04 | -3.44 | -3.48 |
| Ti | 1.76 | -2.72 | 0.12 | -3.42 | -3.54 |
| Ti | 1.78 | -2.62 | 0.19 | -3.38 | -3.57 |
| Ti | 1.8 | -2.52 | 0.26 | -3.34 | -3.6 |
| Ti | 1.82 | -2.43 | 0.33 | -3.28 | -3.61 |
| Ti | 1.84 | -2.34 | 0.39 | -3.23 | -3.62 |
| Ti | 1.86 | -2.26 | 0.46 | -3.16 | -3.62 |
| Ti | 1.88 | -2.18 | 0.52 | -3.09 | -3.61 |
| Ti | 1.9 | -2.1 | 0.58 | -3.01 | -3.59 |
| Ti | 1.92 | -2.02 | 0.64 | -2.94 | -3.58 |
| Ti | 1.94 | -1.95 | 0.7 | -2.85 | -3.55 |
| Ti | 1.96 | -1.88 | 0.75 | -2.77 | -3.52 |
| Ti | 1.98 | -1.82 | 0.81 | -2.68 | -3.49 |
| Ti | 2 | -1.75 | 0.86 | -2.59 | -3.45 |
| Ti | 2.02 | -1.69 | 0.92 | -2.49 | -3.41 |
| Ti | 2.04 | -1.63 | 0.97 | -2.4 | -3.37 |
| Ti | 2.06 | -1.58 | 1.02 | -2.3 | -3.32 |
| Ti | 2.08 | -1.52 | 1.07 | -2.21 | -3.28 |
| Ti | 2.1 | -1.47 | 1.11 | -2.11 | -3.22 |
| Ti | 2.12 | -1.42 | 1.16 | -2.01 | -3.17 |
| Ti | 2.14 | -1.37 | 1.21 | -1.91 | -3.12 |
| Ti | 2.16 | -1.33 | 1.25 | -1.81 | -3.06 |
| Ti | 2.18 | -1.28 | 1.3 | -1.71 | -3.01 |
| Ti | 2.2 | -1.24 | 1.34 | -1.61 | -2.95 |
| Ti | 2.22 | -1.2 | 1.38 | -1.52 | -2.9 |
| Ti | 2.24 | -1.16 | 1.42 | -1.42 | -2.84 |
| Ti | 2.25 | -1.14 | 1.44 | -1.37 | -2.81 |
| Ti | 2.26 | -1.12 | 1.46 | -1.32 | -2.78 |
| Ti | 2.28 | -1.09 | 1.5 | -1.22 | -2.72 |
| Ti | 2.3 | -1.05 | 1.54 | -1.13 | -2.67 |
| Ti | 2.32 | -1.02 | 1.58 | -1.03 | -2.61 |
| Ti | 2.34 | -0.98 | 1.62 | -0.94 | -2.56 |
| Ti | 2.36 | -0.95 | 1.65 | -0.85 | -2.5 |
| Ti | 2.38 | -0.92 | 1.69 | -0.75 | -2.44 |
| Ti | 2.4 | -0.89 | 1.72 | -0.66 | -2.38 |
| Ti | 2.42 | -0.86 | 1.76 | -0.57 | -2.33 |
| Ti | 2.44 | -0.84 | 1.79 | -0.48 | -2.27 |
| Ti | 2.46 | -0.81 | 1.82 | -0.39 | -2.21 |
| Ti | 2.48 | -0.78 | 1.86 | -0.31 | -2.17 |
| Ti | 2.5 | -0.76 | 1.89 | -0.22 | -2.11 |

| Table S4. Raw data from Figure 7. Oxygen sites are indexed based on below cube file (zero-based indexing). The asterisk indicates that atom was used as reference in Figure 7. N/A values indicate that either H could not be stabilized at that site or that significant distortion occurred upon vacancy formation. All energies are in units of eV. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Atom Index |  | ΔEO–ΔEOH | ΔEO | ΔEOH |
| 9 | -3.98 | 0.04 | -3.63 | -3.67 |
| 15 | -3.29 | 1.11 | N/A | N/A |
| 16 | -3.69 | 0.5 | -2.67 | -3.17 |
| 17 | -4.05 | 0.33 | -3.18 | -3.51 |
| 18 | -4.05 | 0.31 | -3.37 | -3.68 |
| 21 | -3.02 | 1.12 | -0.87 | -1.99 |
| 22 | -2.84 | 1.44 | -0.71 | -2.15 |
| 23 | -3.59 | 0.7 | -1.8 | -2.5 |
| 24 | -3.8 | 0.58 | -2.56 | -3.14 |
| 34 | -3.5 | N/A | -3.62 | N/A |
| 40* | -2.95 | 1.26 | -0.84 | -2.1 |
| 41 | -3.5 | 0.6 | -2.37 | -2.97 |
| 42 | -4.03 | 0.4 | -2.74 | -3.14 |
| 43 | -3.89 | 0.58 | -3.12 | -3.7 |
| 45 | -3.18 | 1.16 | -0.71 | -1.87 |
| 46 | -2.98 | 1.43 | -0.99 | -2.42 |
| 47 | -4.01 | 0.38 | -2.28 | -2.66 |
| 57 | -4.07 | -0.02 | -3.27 | -3.25 |
| 63 | -3.01 | 1.1 | -0.9 | -2 |
| 64 | -3.62 | N/A | N/A | N/A |
| 65 | -3.81 | 0.41 | -2.36 | -2.77 |
| 66 | -3.83 | 0.55 | -2.66 | -3.21 |
| 68 | -2.72 | 1.67 | -0.24 | -1.91 |
| 69 | -3.04 | 1.39 | -0.83 | -2.22 |
| 70 | -3.6 | 0.57 | N/A | N/A |
| 75 | -3.95 | 0.2 | -2.96 | -3.16 |
| 76 | -4.05 | 0.07 | -3.52 | -3.59 |
| 77 | -4.04 | -0.12 | -3.49 | -3.37 |
| 79 | -3.43 | 1.27 | -2.1 | -3.37 |
| 84 | -3.2 | N/A | -2.76 | N/A |
| 85 | -4.07 | 0.13 | -3.45 | -3.58 |
| 86 | -4.05 | -0.03 | -3.48 | -3.45 |
| 88 | -3.2 | 1.36 | -1.99 | -3.35 |
| 93 | -3.7 | 0.4 | -2.4 | -2.8 |
| 94 | -4.06 | 0.19 | -3.41 | -3.6 |
| 95 | -4 | 0.11 | -3.52 | -3.63 |
| 97 | -3.12 | 1.32 | -1.47 | -2.79 |
| 98 | -3.43 | 1.24 | N/A | N/A |
| 99 | -2.97 | 1.4 | -0.34 | -1.74 |

Cube file from ASE, written on Thu Jul 26 11:14:17 2018

OUTER LOOP: X, MIDDLE LOOP: Y, INNER LOOP: Z

102    0.000000    0.000000    0.000000

2    9.124825    0.000000   -3.041608

2   -6.083217    8.883757    0.000000

2    0.000000    0.000000   24.599318

8    0.000000    0.000000    0.000000   10.190559

44    0.000000    6.083217    5.922504   12.556670

44    0.000000    0.000000    2.961252   12.556670

8    0.000000    6.083217    5.922504   16.273776

8    0.000000    0.000000    5.922504   14.922782

8    0.000000    8.449329    2.961252   12.556670

8    0.000000    3.717105    2.961252   12.556670

44    0.000000    0.000000    5.922504   18.639887

44    0.000000    6.083217    2.961252   18.639887

8    0.000000    0.020169    5.936628   22.346910

8    0.000000    6.083217    5.922504   21.005999

8    0.000000    2.366112    2.961252   18.639887

8    0.000000   -2.366112    2.961252   18.639887

44    0.000000    6.104394    5.835716   24.800288

44    0.000000    0.009099    2.969111   24.732849

8    0.000000    6.256361    5.469989   28.369509

8    0.000000    0.004462    5.902492   27.114457

8    0.000000    8.476165    2.884043   24.615430

8    0.000000    3.711831    2.920523   24.666082

44    0.000000    0.094800    6.234430   30.962466

44    0.000000    6.247030    3.085939   31.035363

8    0.000000    0.227972    5.804613   34.117762

8    0.000000    6.151017    4.811721   33.731556

8    0.000000    2.463114    3.039631   30.528411

8    0.000000   -2.275504    2.960901   30.724514

8    0.000000    0.000000    5.922504   10.190559

44    0.000000    6.083217   11.845009   12.556670

44    0.000000    0.000000    8.883757   12.556670

8    0.000000    6.083217   11.845009   16.273776

8    0.000000    0.000000   11.845009   14.922782

8    0.000000    8.449329    8.883757   12.556670

8    0.000000    3.717105    8.883757   12.556670

44    0.000000    0.000000   11.845009   18.639887

44    0.000000    6.083217    8.883757   18.639887

8    0.000000    0.023146   11.863051   22.355459

8    0.000000    6.083217   11.845009   21.005999

8    0.000000    2.366112    8.883757   18.639887

8    0.000000   -2.366112    8.883757   18.639887

44    0.000000    6.112012   11.856737   24.989845

44    0.000000    0.001830    8.919087   24.737122

8    0.000000    6.306084   12.077591   28.173266

8    0.000000   -0.020637   11.855499   26.970701

8    0.000000    8.395232    8.867458   24.707688

8    0.000000    3.745397    8.875580   24.793638

44    0.000000    0.009882   12.017943   31.323039

8    0.000000   -0.351290   11.798068   34.465874

8    0.000000    2.235190    9.149101   30.563128

8    0.000000   -2.340488    8.965006   30.653366

8    0.000000    0.000000   11.845009   10.190559

44    0.000000    6.083217   17.767513   12.556670

44    0.000000    0.000000   14.806261   12.556670

8    0.000000    6.083217   17.767513   16.273776

8    0.000000    0.000000   17.767513   14.922782

8    0.000000    8.449329   14.806261   12.556670

8    0.000000    3.717105   14.806261   12.556670

44    0.000000    0.000000   17.767513   18.639887

44    0.000000    6.083217   14.806261   18.639887

8    0.000000    0.016819   17.783514   22.308114

8    0.000000    6.083217   17.767513   21.005999

8    0.000000    2.366112   14.806261   18.639887

8    0.000000   -2.366112   14.806261   18.639887

44    0.000000    6.155716   17.875977   24.905536

44    0.000000   -0.005702   14.830220   24.738954

8    0.000000    6.574743   17.655211   28.061093

8    0.000000    0.046879   17.749611   27.098218

8    0.000000    8.463144   14.784980   24.388673

8    0.000000    3.736955   14.877155   24.661872

44    0.000000    0.248246   18.099669   31.102622

8    0.000000   -0.386279   17.517187   34.225798

8    0.000000    2.185922   14.695701   30.895549

8    0.000000   -2.347202   14.744216   30.521899

8    0.000000   12.166434    0.000000   10.190559

44    0.000000   12.166434    2.961252   12.556670

8    0.000000   12.166434    5.922504   14.922782

44    0.000000   12.176122    5.907922   18.622178

8    0.000000   12.171254    5.948017   22.293503

8    0.000000   14.549143    2.957047   18.632685

8    0.000000    9.811294    2.961911   18.642284

44    0.000000   12.205271    2.958366   24.714385

8    0.000000   12.202331    5.937506   26.743986

8    0.000000   12.166434    5.922504   10.190559

44    0.000000   12.166434    8.883757   12.556670

8    0.000000   12.166434   11.845009   14.922782

44    0.000000   12.170339   11.812498   18.587515

8    0.000000   12.136916   11.742017   22.240918

8    0.000000   14.546013    8.866729   18.593062

8    0.000000    9.799578    8.863360   18.592448

44    0.000000   12.128195    8.904981   24.776938

8    0.000000   12.154481   11.844355   26.786672

8    0.000000   12.166434   11.845009   10.190559

44    0.000000   12.166434   14.806261   12.556670

8    0.000000   12.166434   17.767513   14.922782

44    0.000000   12.176483   17.739363   18.597593

8    0.000000   12.184264   17.649043   22.216272

8    0.000000   14.536401   14.784557   18.589869

8    0.000000    9.801092   14.790487   18.581268

44    0.000000   12.187318   14.809104   24.610368

8    0.000000   12.167029   17.619792   26.758928

8    0.000000    9.848146    2.688277   30.638296

8    0.000000    2.896729   19.874233   30.866190

8    0.000000    0.000000   -0.000000   14.922782

8    0.000000    6.083217   -0.000000   16.273776

1.000000e+00

1.000000e+00

1.000000e+00

1.000000e+00

1.000000e+00

1.000000e+00

1.000000e+00

1.000000e+00

References

[1]	P. Giannozzi, S. Baroni, N. Bonini, M. Calandra, R. Car, C. Cavazzoni, D. Ceresoli, G.L. Chiarotti, M. Cococcioni, I. Dabo, A. Dal Corso, S. de Gironcoli, S. Fabris, G. Fratesi, R. Gebauer, U. Gerstmann, C. Gougoussis, A. Kokalj, M. Lazzeri, L. Martin-Samos, N. Marzari, F. Mauri, R. Mazzarello, S. Paolini, A. Pasquarello, L. Paulatto, C. Sbraccia, S. Scandolo, G. Sclauzero, A.P. Seitsonen, A. Smogunov, P. Umari, R.M. Wentzcovitch, QUANTUM ESPRESSO: a modular and open-source software project for quantum simulations of materials., J. Phys. Condens. Matter. 21 (2009) 395502. doi:10.1088/0953-8984/21/39/395502.

[2]	S.R. Bahn, K.W. Jacobsen, An object-oriented scripting interface to a legacy electronic structure code, Comput. Sci. Eng. 4 (2002) 56–66. doi:10.1109/5992.998641.

[3]	J. Wellendorff, K.T. Lundgaard, A. Møgelhøj, V. Petzold, D.D. Landis, J.K. Nørskov, T. Bligaard, K.W. Jacobsen, Density functionals for surface science: Exchange-correlation model development with Bayesian error estimation, Phys. Rev. B. 85 (2012) 235149. doi:10.1103/PhysRevB.85.235149.

[4]	K.F. Garrity, J.W. Bennett, K.M. Rabe, D. Vanderbilt, Pseudopotentials for high-throughput DFT calculations, Comput. Mater. Sci. 81 (2014) 446–452. doi:10.1016/j.commatsci.2013.08.053.

[5]	C.F. Dickens, J.K. Nørskov, A Theoretical Investigation into the Role of Surface Defects for Oxygen Evolution on RuO 2, J. Phys. Chem. C. 121 (2017) 18516–18524. doi:10.1021/acs.jpcc.7b03481.

[6]	J.H. Montoya, A.D. Doyle, J.K. Nørskov, A. Vojvodic, Trends in adsorption of electrocatalytic water splitting intermediates on cubic ABO 3 oxides, Phys. Chem. Chem. Phys. 20 (2018) 3813–3818. doi:10.1039/C7CP06539F.

[7]	B. Hammer, L. Hansen, J. Nørskov, Improved adsorption energetics within density-functional theory using revised Perdew-Burke-Ernzerhof functionals, Phys. Rev. B. 59 (1999) 7413–7421. doi:10.1103/PhysRevB.59.7413.

[8]	Catalysis-Hub, (2018). https://www.catalysis-hub.org/publications/DickensElectronic2018.